| College of Agriculture and Life Sciences Elson Shields Department of Entomology Cornell University 4144 Comstock Hall Ithaca, NY 14853-2601 | Telephone: 607 255-8428 Fax: 607 255-0939 Email: es28@cornell.edu |
| --- | --- |

Public Aircraft Statement:

Airworthiness of the current UAV design

Pedigree of the design

The design of the atmospheric profiling UAV is a hybrid between the popular full scale cub which has been flown extensively is a reduced scale RC model and the popular Senior Telemaster ARF used by some projects.  The basic platform for the atmospheric profiling UAV is a 33% ARF cub, utilizing a Clark Y airfoil and has been fitted with a lifting horizontal stabilizer is aligned with a negative 1.5 degrees incidence relative to the wing.  This alignment contributes to the stability of the design by lifting the nose of the aircraft.  The presence of the lifting airfoil in the horizontal stabilizer allowed for the movement of the aircraft CG to be moved back to 38% of wing chord, allow for more convenient cargo loading and retain the stability qualities of the high-wing “trainer” design.

Testing and flight history of the design

This UAV was designed and test flown in 1999.  Since the initial flights, the fleet of UAVs utilizing this design has grown to three airframes which have accumulated a total of 300 hours since the initial 1999 test flight.  The airframes have performed without problems or failures.

Airworthiness certification of aircraft

All three airframes were modified by skilled builders and all three airframes have been flown numerous times without any structural issues or problems.  Each airframe is completely inspected annually before being certified for flight by the project leader (E. J. Shields).  All electronic components are installed by a single trained individual to insure uniform installations across all flying UAVs.

Prior to each flight, all electronic systems onboard the aircraft are checked for function and resistance to RF interference.  A flight transmitter range check with antenna collapsed with the engine running at full throttle is conducted to make sure that no RF interference is present before the first flight.  Initial flights after the annual structural inspection are conducted by the in-house test pilot (Project Leader – E. J. Shields).  After the test flights are completed satisfactorily, the airframe is cleared for research flights.  A log book is kept for each of the individual airframes.  Entries are made regarding hours flown, problems which need to be resolved and any other issues necessary.