Visual surveillance:

During the takeoff/landing phase of the flight, the pilot located outside the mobile flight control center will be accompanied by an observer.  The observer’s sole task is to watch for full scale aircraft and other problems during the takeoff/landing procedure and advise the pilot about their presence.

After the control of the UAV has been transferred to the pilot located inside the mobile flight control center who’s task is to monitor the UAV flight from 300 agl to 3000 agl, the takeoff/landing pilot and the observer will remain outside the mobile flight control center and watch for full scale aircraft.  At any point, either pilot can take control of the aircraft to avoid full scale aircraft.

On board camera:

The UAV is equipped with a forward looking camera and the image from the camera is transmitted to the mobile flight control center.  The PIC will be monitoring the flight, in part with the visual image from the front window of the UAV.  Should a full scale aircraft be spotted, the PIC can take direct control of the UAV and follow avoidance procedures.