TSO components:

Engine:  3W 50 cc gasoline engine

Receivers:

Flight Receivers:

Receiver 1 is a FMA Quantum with 8 channels.  This is a PPM receiver with exceptional ability to reject interference and recovers quickly from RF interference.  Receiver 2 is a FMA FS-8 with eight channels.  This receiver is a PPM receiver with the capability of holding the UAV in a circle when the link to the transmitter is lost.  While the FS-8 has good rejection of interference, once it freezes, it reboots slowly.  The combination of these two receivers is the best of both worlds.  One receiver has superior interference rejection and recovers quickly and the second receiver has the ability to fly the UAV in a circle during a lost link issue.  With each receiver flying 50% of the UAV, most interference episodes are unnoticed and survived.  Receivers are available for flights on either 50 MHz (preferred ham band) or 72 MHz (public RC frequencies)

Aerial Sampler Receiver:  FMA M5 on 72 MHz  (channel 13, 29 or 30)

Transmitter (Flight Control):  Airtronics model 660 computer based transmitter.

Telemetry Downlink:

The telemetry downlink is designed around a 1.5 watt ATV transmitter (TXA-5) designed and sold by PC Electronics.  The transmit frequency is either  426, 434 or 439 MHz depending on which of the UAVs are flying.  Information from the GPS antenna is written onto the video signal using a text overlay board (OSD-GPS).  The UAV is equipped with a turnstile antenna to improve reception regardless of the UAV orientation.  The receiving antenna on the Flight Control Center is a 108” vertical whip tuned with a tuner to receive 426-439 MHz.  The signal is passed through a band pass filter to remove unwanted RF and amplified using a TVC-4S down-converter.  The signal is then fed into a standard TV where the information is displayed.

Battery Power:

All battery power on the UAV is supplied by a series of Nicad rechargeable  batteries.  While there is new battery technology which is lighter while providing the same capacity, we choose to remain with the Nicad technology because of their long history of dependability, resistance to vibration and current draw capacity.  All batteries are cycled on a regular basis to prevent the formation of “memory” typical of Nicads.  The cycling also documents the battery capacity, so failing batteries can be detected early and reduce the probability of failing in flight.  Each UAV carries seven 1700 mah battery packs aloft.  One pack is used for engine ignition, each receiver has its own battery pack (2), two packs are used to power the telemetry transmitter and GPS antenna, one battery pack to operate the aerial samplers and one pack to operate the data logger.  Many hours flight experience with this airframe and mission has shown that this allocation of battery power prevents losses from battery overloads and failure.

GPS antenna:  Garmin 35

Onboard data logger:

The onboard data logger is built around an Onset TFX-11 microcomputer.  Altitude and airspeed are sensed using Motorola pressure sensors.  GPS information is supplied by the Garmin 35 gps antenna.  Software was written for the application by the project leader (E. J. Shields).