Lost Voice Communications:

The primary system for voice communications between the aircrew, ground crew and the flight control center is a UHF 2-way radio system.   The flight control center is equipped with a base radio system and all personnel equipped with hand-held units set to the same frequency.  The flight control center is also equipped with a VHF backup system and VHF hand-held units are also available for all personnel.  In the case of a voice communication failure, voice communications will be shifted to the VHF backup system.

Lost Video Link Communications:

While the primary pilot will be monitoring the flight inside the flight control trailer through the onboard mounted camera/video down link, a second pilot will be stationed outside and will also be monitoring the UAV flight.  In the event of a lost video link, the pilot stationed outside the control center will take command of the UAV and return the UAV to the ground.

Lost UAV Control communications:

See procedures under Lost Link Procedures