##### Draganflyer X6 Helicopter Description/Technical Specifications

## The Draganflyer X6 is a remotely operated, unmanned, miniature, vertical take off and land, aircraft designed to carry wireless video cameras and still cameras.

## Helicopter Size

- Dimensions

  - Width: 91cm (36in)

  - Height: 25.4cm (10in)

  - Length: 85cm (33in)

  - Top Diameter: 99cm (39in)

## Weight & Payload

- Helicopter Weight: 35oz (1,000g)

- Payload Capability: 18oz (500g)

- Maximum Gross Take-Off Weight: 53oz (1,500g)

## Materials

- Carbon Fiber

- Glass Filled Injected Nylon

- Aluminum & Stainless Steel Fasteners

- RoHS Compliant

## Landing Gear

- Installed Height: 18cm (7in)

- Stance Width: 30cm (12in)

- Skid Length: 30cm (12in)

- Landing Gear Material: Molded Carbon Fiber

## Rotor Blades

- Three Counter-Rotating Pairs (Six Rotors Total)

- Rotor Blade Material: Molded Carbon Fiber

- Upper Rotor Diameter: 40cm (16in)

- Lower Rotor Diameter: 38cm (15in)

- Upper Rotor Weight: 12g (0.42oz)

- Lower Rotor Weight: 11g (0.38oz)

## Position Navigation Lights

- Type: 1 Watt LED Variable Brightness Emitters

- Luminous Flux at Full Brightness: 40lm

- Purpose: Helicopter Orientation Confirmation

- Visible Condition Range: Full Darkness to Direct Sunlight

- Standard Aircraft Colors

  - Red: Left

  - Green: Right

  - White: Tail/Rear

## Electric Motors

- Brushless Motors: 6

- Configuration: Direct Drive (One Motor per Rotor)

- Safety Features: Stall Protection

- Ball Bearing: 2 per Motor

- Rotor Mounting Points: Integrated

- Nominal Current Draw Per Motor: 1.04 Amps

- Nominal Power / Motor: 15.4 Watts

- Nominal Total Helicopter Motor Power: 92.4 Watts

- Peak Total Helicopter Motor Power: 450 Watts

- Motor Speed at Hover: 2000 RPM

- Voltage: 14.8V nominal

- Weight: 1.34oz (38g)

## Operating Requirements

- Operating Temperature: 14° to 104°F (-10° to 40°C)

- Relative Humidity: 0% to 90% Noncondensing

- Maximum Wind speed: 18mph (30km/h)

## Flight Characteristics:

- Climb Rate: 23ft/s (7m/s)

- Descent Rate: 13ft/s (4m/s)

- Turn Rate: 90°/second

- Cruise Speed: 6mph (10km/h)

- Maximum Speed: 30mph (50km/h)

- Minimum Speed: 0mph (0km/h)

- Launch Type: VTOL (Vertical Take Off and Landing)

- Maximum Altitude ASL: 8,000ft (2438m)

- Maximum Flight Time: 25 minutes

## 11 Onboard Sensors

- 3 Solid State MEMS (Micro-Electro-Mechanical Systems) Gyros

- 3 Solid State MEMS (Micro-Electro-Mechanical Systems) Accelerometers

- 3 Magnetometers (Magneto resistive Sensors)

- 1 Barometric Pressure Sensor

- 1 GPS Receiver

  - GPS Battery Backup: 75mAh Lithium Polymer

## GPS

- GPS Used For: Position Hold, Location & Velocity Data

- Maximum Satellites Tracked Simultaneously: 16

- Position Update Rate: 4 Hz

- GPS Antenna: Ceramic Patch

- Battery Backup: Lithium Polymer

## Black Box Data Recorder

- Flight Data Recording: On-Board

- Stored To: Removable 1Gb MicroSD Memory Card

- Data Recorded: Onboard Sensor Flight Data (Link quality, Orientation, Altitude, Speed, Direction)

## RF Communications

- 2.4GHz Data Link

  - Link Type: Helicopter to Ground & Ground to Helicopter (Two-Way)

  - Helicopter Antenna: Wired Whip Antenna

  - Controller Antenna: Omni-Directional

  - Transmission Power (North America): 100mW (+20dBm)

  - Transmission Power (International): 10mW (+10dBm)

  - RF Data Rate: 250kbps

  - Receiver sensitivity: -100dBm

  - Transmission Technique: DSSS (Direct Sequence Spread Spectrum)

  - Frequency band: 2.4000 - 2.4835 GHz

  - Certifications: CE, FCC, IC, ETSI

  - Data Link Channel Selection: Automatic (12 Channels)

- 5.8GHz Video Link

  - Link Type: Helicopter to Ground (One-Way)

  - Transmitter Antenna: Omni-Directional

  - Receiver Antennas: Omni-Directional & Flat Patch

  - Transmission Power: 12dBm

  - Transmitter Power Consumption: 500mW

  - NTSC and PAL Compatible

  - 7 Selectable Channels: 5740MHz, 5760MHz, 5780MHz, 5800MHz, 5820MHz, 5840MHz, 5860MHz

## Rechargeable Helicopter Battery

- Cell Chemistry: Lithium Polymer

- Voltage: 14.8V nominal

- Capacity: 2600mAh

- Cell Configuration 4s2p (4-series 2-parallel)

- Connectors: Integrated Balance and Power

- Recharge Time: 30 minutes (after typical flight)

- Length: 7.5cm (2.9in)

- Width: 6.7cm (2.6in)

- Height: 2.7cm (1.0in)

- Weight: 228g (8.0oz)

## Camera Attachments

- 10MP (Mega-Pixel) Digital Still Camera with Remote Controlled Tilt, Zoom & Shutter

- 1080p HD (High Definition) Video Camera with Remote Controlled Tilt

- Thermal FLIR (Forward Looking Infra-Red) Camera with Remote Controlled Tilt

- Low Light (0.001lux) Dusk/Dawn Black & White Video Camera with Remote Controlled Tilt