Ground Control Station (GCS)

The Aqua Puma Air Vehicle is controlled by the GCS. The GCS is operated by the MO, while the VO maintains control of the Air Vehicle. Single-person crew operation is not recommended. If operation

by a single-person crew is required, the GCS allows access to and control over all data and input via a single Hand Controller.

NOTE

Monitor GCS battery voltage to ensure adequate power remains. Failure to maintain adequate battery

voltage will result in Loss of Link (LOL).

The basic GCS package, containing a single Hand Controller, is packaged in a waterproof pouch and weighs approx. 8.1 lb. (13 lb. with laptop included). The GCS is equipped with two battery connectors to

allow continuous operation, even while switching batteries. The GCS will operate for approx. 1.5 - 2.0 hours on a single BB-2557 battery.  The GCS can also be used with the common BA-5590, BB-390B/U,

and BB-2590/U batteries.

GCS in Pouch

Set Up

A second GCS can be used as an RVT. The RVT is used to view real-time video from any location within range of the Air Vehicle. To operate a GCS as an RVT, leave the uplink antenna disconnected and

the transmitter off.