Emergency Procedures

Emergencies are handled in the same manner as any manned aircraft.  An IFE will be declared with ATC via radio or telephone and the PIC will immediately take actions directed by emergency checklists. Aircraft recovery will normally be via the most direct route to Restricted Airspace avoiding populated areas enroute to Creech AFB or as directed by ATC. The normal Restricted Airspace entry point is the INS 100/16 which is the southeast corner of R4806. The RPA will remain over unpopulated areas at all times and, as with manned aircraft, comply with ATC instructions to the extent the emergency allows.

The preferred routing is the same as the lost link routing:

After leaving R4806 while in the MOA: From anywhere in the MOA it will go direct to the INS 057R 27 mile fix, direct the INS 100R 16 mile fix which is in R4806.  This routing will keep the aircraft in the MOA and Restricted airspace.

After leaving R4806 while in the SETB: The aircraft will return to R4806 via direct the INS 100R 16 mile fix which is in restricted airspace.