Lost Voice Communications Procedures

The MQ-1 pilot has more reliable communication capabilities from their ground station than any airborne aircraft because they are not reliant upon the aircrafts electrical systems working.  The MQ-1 has a UHF/VHF/FM radio on board that the pilot uses for communication with Air Traffic Control (ATC).  They also have UHF/VHF/FM radios in the ground station, which provide limited communication capability, and telephone land lines to ZAB, Cannon Tower, and Cannon Approach Control.

In the event that all direct voice radio communications between the pilot and ATC are lost, the pilot will command MQ-1’s transponder to a discrete Lost Communication code of 7601 and the pilot will contact ATC by the direct telephone landline and advise that radio communication has been lost.  The Pilot will request via telephone to return to Cannon AFB and land.

If a loss of radio communications is experienced on the ground, pilot will contact Cannon Tower ATC via landline and wait for further instructions.  Tower ATC will not permit departure until radio communications is restored.