Lost Data Link Procedures

- General:

  - Due to redundant systems it is highly unlikely for the MQ-1 to lose command link with its associated ground control station (GCS).   However, in the event of loss of control link between the aircraft and the launch and recovery element’s (LRE’s) GCS when airborne, the MQ-1 aircrew will follow the unit’s local lost link checklist procedures.

  - All Lost Link Emergency Missions and associated lost link altitudes will be deconflicted among all UAS aircraft.  The lost link altitudes will be at 1000’ increments when operating in within the Main MQ-1 Operating Area (Sumner Air Traffic Control Assigned Airspace (ATCAA), R-5104, R-5107, and the R-5104 to R-5107 corridor.  All Emergency Missions will terminate within the lateral boundaries of R-5104.

  - At the initial lost link indication the pilot will attempt to regain link of the aircraft IAW TO1Q-1(M)B-1:

(1) Attempt to regain link by switching transmitters and antennas.

(2) Attempt to regain link by using alternative primary and secondary frequencies.

(3) Attempt to regain link by executing climbing and turning maneuvers.

(4) If control data link is not reestablished, the pilot will notify ATC that lost link has

occurred and that the aircraft is squawking 7701 with the following information:

(a) Time of lost link

(b) Last known position

(c) Altitude

(d) Direction of flight

Figure 1. Main MQ-1 Orbit Area and Melrose Range (R-5104A/B).

2. Lost link procedures within the Main MQ-1 operating area and R-5104A/B:

  - Operations within the Main MQ-1 operating area will be from FL180 to FL230.  This area except for the extensions east of R-5104 and north of Sumner Muni are within the Sumner ATCAA.

  - After initial indication of lost link and the aircraft is unable to regain link, the aircraft’s Emergency Lost Link Mission will conduct the following:

    - Aircraft will orbit in a 1.5 NM circular orbit for 30 minutes within the cleared airspace given by ATC and at last assigned altitude (within FL180 and FL230).

    - If still unable to obtain link within 30 minutes, the aircraft will proceed to the Melrose restricted area (R-5104A/B) at the same cleared altitude and on a standardized emergency mission routing.

    - Lost link aircrew will contact ZAB or Cannon Approach and notify them of the lost link condition.  ZAB or Cannon Approach will notify all aircraft within the Sumner ATCAA and other affected airspace of the lost link aircraft.  Information will include lost link aircraft’s position, altitude, and direction of flight.

    - When lost link aircraft arrives in Melrose restricted area (R-5104A/B), the aircraft will enter a six waypoint lost link orbit (3 NM radius centered on N 34° 15' 16.00" W 103° 47' 50.00", see figure 1).  This six waypoint lost link orbit will provide a sterilized area in the event link cannot be re-established.  The aircraft will orbit in the last six waypoints until link is regained or fuel exhaustion.  At fuel exhaustion, the MQ-1 will lower the landing gear and continue to fly the orbit while descending (engine out) until ground impact.

2.3  If a lost link situation occurs while operating within R-5104, the  aircraft will orbit in a 1.5 NM circular orbit for 30 minutes at the last operating altitude.

2.3.1	If still unable to obtain link within 30 minutes, the aircraft will proceed to the R-5104 lost link orbit area and hold until link is re-established or fuel exhaustion.

Figure 2. Corridor between R-5104 and R-5107.

3.  Lost link procedures within corridor and R-5107:

  - When operating within the corridor all MQ-1 aircraft will fly within FL180 through FL210.

  - When operating within the corridor and command link is lost, the MQ-1 will hold for 30 minutes at the last assigned altitude within a 1.5 NM radius while the pilot attempts to re-establish link.  The pilot will ensure that the MQ-1’s initial lost link orbit remains within the corridor COA airspace (corridor is 8 NMs wide).   If unable to regain link within the 30 minute hold period, the lost link aircraft will proceed to R-5104 A/B on a standardized lost link Emergency Mission route at the last assigned altitude.  Upon reaching R-5104 A/B, the lost link aircraft will conduct 2.2.4 (above) at its last assigned altitude.

  - If link is lost while within R-5107, the pilot will ensure that the initial holding area(s) remains within R-5107.  If aircrew cannot regain link with the aircraft after the first indication of lost link the aircraft will hold in a 1.5 NM orbit for 30 minutes at the last assigned altitude.  If unable to regain link within the 30 minute hold period, the lost link aircraft will proceed to R-5104 A/B on a standardized lost link Emergency Mission route at the last assigned altitude.  The aircraft will continue to R-5104 A/B at the altitude it was flying within the corridor and conduct 2.2.4 (above).