Figure 1-1 Puma AE Air Vehicle

Table 1-2 Puma AE Air Vehicle Characteristics

The Puma AE Small Unmanned Aerial Vehicle (SUAV) is a hand-launched reconnaissance and surveillance tool. The system transmits live airborne video images, compass headings, and location information to a Ground Control Station (GCS), including laptop if connected,

and a GCS used as a Remote Video Terminal (RVT). This capability enables operators to navigate, search for targets, recognize terrain, and record all information for analysis. Puma AE provides operational capability in the following areas:

• Remote reconnaissance and surveillance

• Force protection

• Convoy security

• Target acquisition

• Battle damage assessment

This operator’s manual provides technical information and guidance for assembly, operation, and maintenance.  The Puma AE Air Vehicle can be launched and recovered in minutes without special equipment. The system employs a self-stabilizing Air Vehicle configuration with stability augmentation avionics, and provides ease of control and steady video imagery. Air and ground components are lightweight. The Puma AE Air Vehicle is battery powered and has low visual, acoustic, and thermal signatures. The system includes one payload per Air Vehicle; each payload houses two electro-optic (EO) and two infrared (IR) cameras. The Puma AE Air Vehicle flies for 2-3 hours on two rechargeable Lithium Ion (Li-ion) battery packs. The Air Vehicle may also be flown with a single battery and a dummy battery for approx. 1 hour. The Puma AE system is operated by a two-person team consisting of a Vehicle Operator (VO) and Mission Operator (MO).

1 PUMA AE SYSTEM DESCRIPTION

The Puma AE Air Vehicle is assembled from seven components. It is broken down for storage and transportation into two Storm Cases.  The Wing Case contains the Center Wing, Left Wingtip, Right Wingtip, and Stabilator. The Fuselage Case contains the Fuselage, Payload (either attached or unattached to fuselage), and Tail-boom.  The Fuselage Case also holds the FRK, spare propeller, and two batteries. A foam liner supports and protects the Air Vehicle and accessories from damage. The assembled Puma AE Air Vehicle is shown in Figure 1-1.

The Puma AE Air Vehicle is assembled from seven components. It is broken down for storage and transportation into two Storm Cases.  The Wing Case contains the Center Wing, Left Wingtip, Right Wingtip, and Stabilator. The Fuselage Case contains the Fuselage, Payload (either attached or unattached to fuselage), and Tailboom.  The Fuselage Case also holds the FRK, spare propeller, and two batteries. A foam liner supports and protects the Air Vehicle and accessories from damage. The assembled Puma AE Air Vehicle is shown in Figure 1-1.