- All “Lost-Link” Procedures shall remain within the PTA designated COA airspace and within clear sight of the Operator or Visual Observer.   Procedures will avoid any assembly of people and surface vehicle traffic (both water and road).

- UA will fly to a predetermined “rally-point” at a predetermined altitude within the system specific visual surveillance stratum:

  - WASP:  500 feet vertically, one-half mile laterally

  - RAVEN:  1200 feet vertically, one mile laterally

  - PUMA AE:  1500 feet vertically, one mile laterally

- The “rally-point” is automatically set 20 meters east of the “home” waypoint, and the “home” waypoint is located at the GCS antenna.   If link is reestablished on the lost-link return flight, the operator will make a determination whether to continue the mission or land the aircraft.  If the aircraft cannot reestablish data link, the UA will continue to the “rally-point” and automatically execute the “auto-land” function at “home”.

- For over-water operations, the GCS will be located on a watercraft, therefore the rally point will continuously move in relation to the watercraft’s movements.

- In the event of a severe malfunction of system failure, the operator has the ability of terminating the engine power to force the UA to “auto-land”.

- Emergency/Lost-Link Procedures will remain at least 1 NM outside the applicable Final Approach fixes for instrument approaches at London-Corbin (KLOZ) and Williamsburg-Whitley (W38) airports.  All UA for these airports.   All operators and observers are required to monitor KLOZ (123.0) and W38 (122.7) CTAF/UNICOM frequencies and announce execution of lost-link procedures and/or emergency fly-away conditions.  The pilot will provide the following:

- UA last known location

- UA altitude

- Direction of flight/heading

- Fuel on board/Battery Time

- Pilots intentions

- Termination of flight/emergency condition

The