| Launch/Recovery Procedures: |  |
| --- | --- |
| Note: In accordance with the WASP and RAVEN UAS Standard Operating Procedures (SOP) UAS flights will be IAW guidance in an FAA approved COA and the SOP. The Raven UAS is a hand-launched system, launch and recovery procedures are listed below. |  |
| A. Launch Procedure: |  |
| 1. Aircraft and Ground Control Unit (GCU) are assembled |  |
| 2. Aircraft battery is inserted and aircraft establishes GPS link and control link with GCU |  |
| 3. Preflight checks are made to ensure proper operation of aircraft and GCU |  |
| 4. Mission controller holds aircraft in a safe direction pointed into the wind |  |
| 5. Pilot throttles electric motor to 100% |  |
| 6. Battery power is checked |  |
| 7. Pilot gives mission controller green light |  |
| 8. Mission controller throws (hand-launched) aircraft into wind and aircraft achieves flight |  |
| B. Recovery Procedure: |  |
| 1. Aircraft is flown back to GCU site |  |
| 2. Altitude is lowered to a safe height |  |
| a. Safe altitude determined by ground obstructions |  |
| b. Landing altitude typically 50 - 400ft AGL |  |
| 3. When aircraft is at landing location/position, UA is put into “Autoland” mode |  |
| a. Power to motor is cut |  |
| b. Horizontal stabilizer deflects fully upward |  |
| c. Aircraft pitches nose up and begins a steep decent to the ground with wings level (typically a 1:1 glide ratio) |  |
| 4. 4. Aircraft impacts ground causing components to separate (i.e. wings, horizontal stabilizer, etc) and absorb the impact |  |
| 5. Aircraft components are gathered up and inspected for damage |  |
| 6. Aircraft is reassembled |  |

Recovery Procedures: